Study Beamforming Concepts: Spatial filtering to enhance signals.
Beamforming is a spatial filtering technique used in antenna arrays to transmit or receive signals directionally rather than broadcasting in all directions. By adjusting the amplitude and phase of individual antenna elements, signals constructively interfere in desired directions and destructively interfere elsewhere, improving signal-to-interference-plus-noise ratio (SINR), increasing capacity, and reducing interference.

Key Beamforming Concepts & Types

- Analog Beamforming: Uses phase shifters to control the analogue signal, which is less complex but typically produces only one beam.

- Digital Beamforming: Applies weight adjustments in the digital domain, offering high flexibility and the ability to create multiple simultaneous beams.

- Hybrid Beamforming: Combines analogue and digital approaches to balance complexity and performance, commonly used in 5G.

- Adaptive Beamforming: Dynamically adjusts antenna patterns in real-time to track users, maximize signals, and minimize interference.

- Massive MIMO: Utilizes a large number of antennas at the base station to generate precise, narrow beams for multiple users simultaneously.

Operational Principles

- Constructive/Destructive Interference: Signals are phase-shifted to align (sum up) at the target receiver and cancel out at unauthorized locations.

- Weighting: Each antenna element's signal is multiplied by a complex weight (amplitude and phase adjustment) to optimize the beam shape.

- Spatial Filtering: Acts as a directional filter that, unlike frequency filters, separates signals based on their angle of arrival.

Applications

- 5G Wireless Networks: Directs radio waves towards users for higher data rates and reduced energy consumption.

- Radar and Sonar: Identifies the direction of targets.

- Medical Imaging: Used in ultrasound for clearer imaging.

Implement delay and sum beamforming: Align signals with delays.

Delay-and-sum beamforming is one of the simplest and most widely used array signal processing techniques. The idea is to align signals received at different sensors (microphones, antennas, etc.) by compensating for propagation delays, then sum them to enhance the desired direction while suppressing others.

### Core Concept

- Suppose you have M sensors arranged in an array.

- A plane wave arrives from direction θ.

- Each sensor receives the signal with a delay depending on its position relative to the wave front.

- By applying the appropriate delay (or phase shift) to each sensor’s signal, you align them in time.

- Summing the aligned signals reinforces the wave from θ and attenuates signals from other directions.

### Mathematical Formulation

For sensor m, the received signal is:

xm(t)=s(t−τm)+nm(t)

where:

- s(t) is the source signal,

- τm is the propagation delay to sensor m,

- nm(t) is noise.

The beamformer output is:

y(t)=∑m=1Mxm(t+τm)

This effectively aligns the signals before summation.

### Implementation Steps

- Array Geometry Define sensor positions (e.g., linear array with spacing d).

- Delay Calculation For a desired steering angle θ, compute delays:

τm=dm⋅sin⁡(θ)c

where dm is the sensor’s position along the array and c is wave speed (e.g., speed of sound).

- Signal Alignment Apply delays to each sensor’s signal. In practice:

  - For continuous-time signals: shift directly.

  - For sampled signals: use interpolation or fractional delay filters.

- Summation Add the delayed signals to form the beamformer output.

Code:

import numpy as np

import matplotlib.pyplot as plt

def fractionaldelay(signal, delaysamples):

N = len(signal)

spectrum = np.fft.fft(signal)

freqs = np.fft.fftfreq(N)

phaseshift = np.exp(-2j * np.pi * freqs * delaysamples)

shifted = np.fft.ifft(spectrum * phaseshift)

return np.real(shifted)

#Parameters

fs = 8000 #Sampling Frequency

duration = 0.01  # Seconds

t = np.arange(0, duration, 1/fs)

c = 343   # Speed of sound (m/s)

d = 0.05  # Sensor spacing (m)

M = 4     # Number of sensors

theta = np.pi/6  # Steering angle (30 degree)

#Source signals

f0 = 1500

s = np.sin(2*np.pi*f0*t)

# Generate signals

signals = np.zeros((M, len(t)))

for m in range(M):

delay = (m*d*np.sin(theta)) / c

delaysamples = delay * fs

signals[m] = fractionaldelay(s, delaysamples)

# Delay and Sum

output = np.zeros(len(t))

for m in range(M):

delay = (m * d * np.sin(theta)) / c

delaysamples = delay * fs

aligned = fractionaldelay(signals[m], -delaysamples)

output += aligned

output /= M

# Plot

plt.figure(figsize=(6,6))

plt.subplot(3,1,1)

plt.title("Original Source Signal")

plt.plot(t, s)

plt.subplot(3,1,2)

plt.title("Signals at sensors (with delays)")

for m in range(M):

plt.plot(t, signals[m], label=f"Sensor {m}")

plt.legend()

plt.subplot(3,1,3)

plt.title("Beamformer output (aligned and summed)")

plt.plot(t, output, color='red')

plt.tight_layout()

plt.show()

### Notes

- Fractional delays: Real-world delays are rarely integer multiples of the sampling period. Use interpolation or FIR fractional delay filters for accuracy.

- Performance: Works well for narrowband signals and small arrays. For wideband signals, more advanced beam formers (e.g., frequency-domain methods) are often used.